# AutoLink RadioSense

Document 03 — PlatformIO Build Environments and Firmware Upload Guide

| Purpose: | Explain how to configure, select, build, and upload different C++ programs to the ESP32 using PlatformIO in VS Code. |
| --- | --- |

# 1. Introduction

The AutoLink RadioSense project contains two main types of software:

- Production software: The main application developed under src/.

- Hardware testing software: Independent programs stored under test/system/ to test individual functions such as LoRa transmission and reception.

Each program has its own C++ source file. We use PlatformIO build environments to tell PlatformIO which source files to compile for a particular task.

This allows us to test individual components without modifying or replacing the production src/main.cpp.

# 2. Project Structure

The relevant project directories are:

AutoLink RadioSense/

├── src/

│   ├── Core/

│   │   ├── LoRa_Manager.cpp

│   │   └── LoRa_Manager.h

│   └── main.cpp

│

├── test/

│   ├── system/

│   │   ├── 1 - Raw Transmission/

│   │   │   └── Raw_Transmission.cpp

│   │   ├── 2 - Raw Reception/

│   │   │   └── Raw_Reception.cpp

│   │   └── 3 - Raw Transmission & Reception/

│   │       └── Raw_Transmission_Reception.cpp

│   └── unit/

│

├── lib/

│   └── LoRa/

│

├── docs/

└── platformio.ini

The src/ directory contains production code, while test/system/ contains independent hardware test programs.

The external LoRa library is stored in lib/. Shared project modules, such as LoRa_Manager.cpp, can be compiled alongside production software or a hardware test when required.

# 3. What Is a Build Environment?

A PlatformIO build environment is a named configuration that defines how a firmware program is built.

For this project, we use four environments:

| Environment | Purpose |
| --- | --- |
| production | Builds the main AutoLink RadioSense application |
| raw_tx | Builds the raw LoRa transmission test |
| raw_rx | Builds the raw LoRa reception test |
| raw_tx_rx | Builds the combined transmission and reception test |

The environment names are defined in platformio.ini.

For example:

[env:production]

defines the production environment, while:

[env:raw_tx]

defines the raw transmission test environment.

Important: An environment does not automatically select a C++ file based on its name. The build_src_filter setting specifies which source files are included in that environment.

# 4. Understanding platformio.ini

The project-level configuration includes:

[platformio]

src_dir = .

default_envs = production

- src_dir = . sets the project root as the source base.

- default_envs = production makes production the default environment when no environment is specified.

The shared configuration is:

[env]

platform = espressif32

board = esp32doit-devkit-v1

framework = arduino

monitor_speed = 115200

These settings apply to all four environments unless overridden.

Each environment then defines its own source filter. For example, the raw transmission environment selects:

[env:raw_tx]

build_src_filter =

+<src/Core/*.cpp>

+<test/system/1 - Raw Transmission/Raw_Transmission.cpp>

The +<...> patterns include matching source files in the build. Exclusion patterns beginning with -<...> prevent matching files from being compiled.

Because the source base is the project root, these filter paths start from AutoLink RadioSense/.

Always check the complete environment configuration to ensure that exclusion patterns do not accidentally remove a required shared module.

# 5. Selecting an Environment in VS Code

## Method A — PlatformIO Project Tasks

1.	Open the AutoLink RadioSense project in VS Code.

2.	Open the PlatformIO sidebar using the PlatformIO icon.

3.	Expand PROJECT TASKS.

4.	Locate the environment you want to use.

5.	Expand its General section.

6.	Select the required task.

For example:

PROJECT TASKS

│

├── production

│   └── General

│       ├── Build

│       └── Upload

│

├── raw_tx

│   └── General

│       ├── Build

│       └── Upload

│

├── raw_rx

│   └── General

│       ├── Build

│       └── Upload

│

└── raw_tx_rx

└── General

├── Build

└── Upload

To build the production firmware, select production → Build.

To build the transmission test, select raw_tx → Build.

To upload the transmission test, select raw_tx → Upload.

Remember: Select the environment first, then choose the operation you want to perform. The top-level Build and Upload toolbar buttons may use the default environment, so use the environment-specific tasks when you need to be certain which program is selected.

# 6. Selecting an Environment Using the Terminal

You can also select an environment explicitly using PlatformIO commands.

Run these commands in the VS Code terminal opened at the project root.

## A. Production Application

Build:

pio run -e production

Upload:

pio run -e production -t upload

## B. Raw Transmission Test

Build:

pio run -e raw_tx

Upload:

pio run -e raw_tx -t upload

## C. Raw Reception Test

Build:

pio run -e raw_rx

Upload:

pio run -e raw_rx -t upload

## D. Combined Transmission and Reception Test

Build:

pio run -e raw_tx_rx

Upload:

pio run -e raw_tx_rx -t upload

The -e option specifies the environment. The -t upload option tells PlatformIO to upload the firmware after building it successfully.

# 7. Opening the Serial Monitor

After uploading the firmware, open the Serial Monitor to view the ESP32's output.

The configured baud rate is 115200.

Run:

pio device monitor

If necessary, specify the environment:

pio device monitor -e raw_tx

Alternatively, use the Serial Monitor task in the PlatformIO interface.

If the monitor is blank, verify the baud rate, check that the program prints messages using Serial.print() or Serial.println(), and reset the ESP32 if startup messages were printed before the monitor connected.

# 8. Recommended Workflow

Always follow this sequence when testing a program:

1.	Select the environment corresponding to the program you want to run.

2.	Build the environment.

3.	Check the terminal output for compilation errors.

4.	Confirm that only the intended program and required shared modules were compiled.

5.	Upload the firmware to the ESP32.

6.	Open the Serial Monitor.

7.	Verify the program's output and record the test results.

For example, to test raw transmission:

pio run -e raw_tx

pio run -e raw_tx -t upload

pio device monitor -e raw_tx

Do not upload the firmware if the build fails.

# 9. Troubleshooting

| Problem | What to check |
| --- | --- |
| The wrong program is compiled | Check the selected environment and its build_src_filter |
| Multiple main() or setup() definitions | Ensure that only one independent application entry point is compiled |
| LoRa.h cannot be found | Verify that the LoRa library exists under lib/LoRa/ |
| A shared module is missing | Ensure the relevant .cpp file is included in the environment's source filter |
| The build succeeds but no output appears | Check Serial Monitor baud rate, serial initialization, and program logic |
| Upload fails | Check the USB cable, COM port, and ESP32 connection |
| An environment is not listed | Save platformio.ini, then refresh or reload PlatformIO in VS Code |

# 10. Important Rules

- Do not replace or rename src/main.cpp just to run a hardware test.

- Do not select multiple independent application entry points in the same environment.

- Keep production code under src/ and hardware test programs under test/.

- Reuse common modules such as LoRa_Manager.cpp where appropriate.

- Build successfully before uploading.

- Use explicit environment names in terminal commands to avoid accidentally building the default environment.

- After modifying source filters, verify the build output to confirm that the intended files are being compiled.

# 11. Quick Reference

| Task | Command |
| --- | --- |
| Build production | pio run -e production |
| Upload production | pio run -e production -t upload |
| Build raw transmission | pio run -e raw_tx |
| Upload raw transmission | pio run -e raw_tx -t upload |
| Build raw reception | pio run -e raw_rx |
| Upload raw reception | pio run -e raw_rx -t upload |
| Build combined test | pio run -e raw_tx_rx |
| Upload combined test | pio run -e raw_tx_rx -t upload |
| Open Serial Monitor | pio device monitor |

Final reminder: The environment determines the build configuration, and the source filter determines which C++ files are compiled. Always verify both when troubleshooting unexpected firmware behavior.